#include <Keypad.h>

#include <Adafruit_LiquidCrystal.h>

#include <Servo.h>

int temp = 0;

int target = 0;

bool state = 0;

bool prev = 0;

int reset = 0;

int master = 0;

Servo servo_11;

Adafruit_LiquidCrystal lcd_1(0);

const byte rows = 4;

const byte cols = 3;

char keys[rows][cols] = {

{'1','2','3'},

{'4','5','6'},

{'7','8','9'},

{'*','0','#'}

};

byte rowPins[rows] = {8, 7, 6, 5};

byte colPins[cols] = {4, 3, 2};

Keypad keypad = Keypad( makeKeymap(keys), rowPins, colPins, rows, cols );

void input()

{

lcd_1.clear();

target = 0;

char key;

while(key != '#')

{

key = keypad.getKey();

if (key == '*')

{

target /= 10;

lcd_1.clear();

lcd_1.print(target);

}

else if (key != NO_KEY && key != '#')

{

target *= 10;

target += int(key) - 48;

if(target > 40)

{

target = 0;

lcd_1.clear();

digitalWrite(9, HIGH);

lcd_1.print("input <= 40");

delay(2000);

digitalWrite(9, LOW);

lcd_1.clear();

}

lcd_1.setCursor(0,0);

lcd_1.print(target);

}

delay(10);

}

}

void setup()

{

servo_11.attach(11, 500, 2500);

servo_11.write(0);

pinMode(A0, INPUT);

pinMode(A1, INPUT);

pinMode(12, INPUT);

pinMode(LED_BUILTIN, OUTPUT);

pinMode(9, OUTPUT);

pinMode(10, OUTPUT);

lcd_1.begin(16, 2);

Serial.begin(9600);

input();

}

void loop()

{

master = (analogRead(A1) - 1013) / 10 ;

if(master == 1)

{

int read = analogRead(A0);

reset = digitalRead(12);

if(reset == HIGH)

{

digitalWrite(10, LOW);

servo_11.write(0);

input();

prev = false;

}

temp = map(((read - 20) * 3.04), 0, 1023, -40, 125);

if(temp <= target - 3)

{

state = true;

}

else if (temp >= target)

{

state = false;

}

lcd_1.setCursor(0,1);

if(state == true)

{

if(prev == false)

{

servo_11.write(90);

delay(1000);

digitalWrite(10, HIGH);

delay(500);

digitalWrite(10, LOW);

}

lcd_1.setBacklight(1);

lcd_1.print(temp);

lcd_1.print(" heating on            ");

digitalWrite(LED_BUILTIN, HIGH);

}

else

{

if(prev==true)

servo_11.write(0);

lcd_1.setBacklight(1);

lcd_1.print(temp);

lcd_1.print(" heating off          ");

digitalWrite(LED_BUILTIN, HIGH);

}

prev = state;

}

else

{

digitalWrite(10, LOW);

if(prev==true)

servo_11.write(0);

lcd_1.setCursor(0,1);

lcd_1.setBacklight(0);

lcd_1.print("power off             ");

digitalWrite(LED_BUILTIN, LOW);

prev = false;

}

delay(100);

}

https://www.tinkercad.com/things/itaQX1HBfqp/editel?sharecode=wNpQA1Ro6J-qL75CJjq72bRsnnAPDJ8QlUn7FUBOC5M